Sponsor: AFRL

Project No.: 144

Contract: ASTRA IST

This document is for authorized use only. Any information contained herein should be considered proprietary unless otherwise specified.

© 2024 Dependable Computing. All Rights Reserved

Dependable Computing

V0.8         11/11/2024

Proposed Behavior Description

Zone Alert Service

Abstract

This document provides behavior descriptions of the zone alert service designed to inform the end user and stakeholders of the services behavior with respect to its provided services.

Changes

| Version | Date | Change |
| --- | --- | --- |
| V0.1 |  | Initial draft |
| V0.2 |  | Updates based on behavior validation from stakeholders |
| V0.3 | 1/30/2024 | Modified behavior specification based on the lack of vehicle radius in route planning, replaced behavior semantics with concept of buffered keep-out and keep-in zones and consistent semantics of zones with Route Planning Service. Fixed typos. Discussed alerts only for zones in a vehicle’s operating region. |
| V0.4 | 1/30/2024 | Clarified when one or both projected or current violation is reported for a given vehicle and zone. |
| V0.5 | 1/31/2024 | Vehicle-Radii behaviors removed. Current and future zone alerts replaced with a single simpler reporting structure where violation time can be zero to indicate current violation. |
| V0.6 | 3/29/2024 | Text cleanup |
| V0.7 | 10/24/2024 | Added context and definitions and clarified some text on expected behaviors |
| V0.8 | 10/28/2024 | Revised behavior for keep-in zones based on solution exploration updates |
| V0.9 | 11/12/2024 | Document cleanup. Removed stale information after requirements/parameters sections. |
| V0.12 | 12/9/2024 | Updated for 2D zone violation semantics. |

- .

Contents

1	Introduction	1

2	Mandate	1

3	Solution Approach	1

3.1	Explanation of Solution Exploration Graph Notation	1

3.2	Apparent Solution Options	3

3.3	Solution Approach	4

4	Resulting Solution Behavior Requirements	6

5	Service Parameters	8

Introduction

The Zone Alert Service is a stand-up service that can be activated as part of an OpenUxAS instance. Its purpose is to alert subscribers when an OpenUxAS vehicle is or may soon be in violation of a zone.

This document describes the technical requirements and interface of the Zone Alert service for the purpose of system design and implementation as well as its verification.

Each requirement defined here should tie to behavior defined in the Zone Alert Behavior document in the Mandate folder. Explicit traceability from behavior description to top level requirements is provided in this document.

Mandate

See the Zone Alert Behaviors document in the Mandate folder.

Solution Approach

This section provides a brief description to the approach to solution for the mandated Zone Alert service.

The definitive solution approach is defined in the Solution Exploration diagram of the Specification/Design folder.

This section contains text to elucidate that diagram as well as provide some notes.

Explanation of Solution Exploration Graph Notation

The Solution Exploration diagram is a Solution Exploration Graph as devised by J. Rowanhill. It is a deviation from the Problem Frame graph concept of Jackson and Hill et al. This format is designed to allow rapid, natural language problem solving while retaining the core “decompose problems first and sew up the solution later” paradigm of Problem Framing.

Part of the graph during some arbitrary iteration o development, for illustrative purposes, is shown in Figure 1. Note: This is not ground truth. The reader must refer to the full diagram for the full and actual development semantics of the latest iteration of the service.

Figure 1. Part of a typical Solution Exploration graph.

The graph consists of ovals representing problems/requirements that must be overcome/achieved in the design. This is decomposed by directed graph edges to further nodes. Problems that are concrete are resolved by solutions, represented as rectangles with a rectangular margin to their left (“machines” in problem framing). Their contained text describes their solution approach.  Simple rectangles represent context information for problems or solutions to which they are attached.

A shape with a triangle pointing out of it is a reference to a remote graph node. This can be used to refer to a common sub-problem that has or will be solved in another graph, or simply to organize graphs into reasonable sizes. A node with a triangle pointing into it is a node that is referenced elsewhere.

Diamonds represent choices, where there is more than one potentially valuable option for solution. Nodes that are crossed out are discontinued. Discontinued problems are recognized as not existing after all. Discontinued solutions are those that have been rejected.

A slash through a node indicates the node has been eliminated from concern. Problems with slashes through them are identified as being incorrect. Solutions with slashes through them are rejected options, either because they are inferior to another approach (under a diamond), or are incorrect and not desired to be corrected.

A rectangle with rounded edges that labels and edge between other nodes represents a “compromise” or “detail”. As sub-problems are solved, they introduce details that may contradict, override, or define required elements of parent solutions. This information flows backwards along the graph’s edges as is important in a problem framing approach

A leaf problem without a defined solution is an unsolved problem. In general, the child of a problem should be a proposed solution with potential sibling sub-problems. However, in some cases a problem is decomposed only into sub-problems when its composite solution is deemed adequate.

The graph represents core truth of analysis and solution development. The text below provides supplementary, secondary notes about the chosen solution and reasoning.

Apparent Solution Options

The zone alert service responds to vehicle state reports by checking for vehicle zone violations at the present time and those that might occur in the future. Computing present violations can be done accurately. Computing future violations is performed without explicit knowledge of a vehicle’s future movement plans except for a route plan. A route plan will never violate zones. Note, however, that (1) a vehicle might not follow its route plan, and (2) at mission points within the plan will explicitly operate off the route.

Three solutions seemed possible for warning about impending (not current) zone violations:

- Provide a service that provides band information, similar to DAIDALUS, allowing one to know how long until a zone violation with any given zone in any given direction a vehicle might fly within some fixed future time window.

- Use the total knowledge of past vehicle position reports, in some lagging time window, to predict a Bezier-curve path along which the vehicle is likely to continue to travel on a fixed future time window.

- Apply the current linear velocity of each vehicle and project it forward in time to predict zone violations (even if the vehicle is accelerating), and assume it is close enough for prediction on short future time windows.

Option 1 seems most useful for general application, and therefore has greatest validity. Option 2 might produce superior results to option 3 in scenarios below where a vehicle skirts along a zone boundary while facing towards it at times. Option 3 seems the most error prone to false positive and false negative imminent zone violation alerts.

There are likely no resource limitations that make options 1 or 2 computational infeasible.

Of the three options, option 3 is possibly the easiest to develop and engineer. Therefore, it is not the solution of greatest validity but is possibly a better simple demonstration service for assurance of correct and appropriate (verified and validated) service.

Under the above reasoning Option 3 was chosen for the example assured service work. Note that using this document and the reasoning of various diagrams and problem definition documentation where development cost is less restricted, one would likely choose Option 1 or Option 2 (or some other superior solution concept) and argue against the satisfactory validity of Option 3.

Solution Approach

The Zone Alert Service computes imminent zone violations by detecting future violations on a fixed, limited time window into the future, along a vehicle’s present instantaneous linear velocity. More detail follows.

### Time Window

The Zone Alert Service computes whether a vehicle may violate a zone in the future. It does this within a time window. A vehicle’s time window is defined as the time from the vehicle’s latest state report, called TS, to a time TW seconds in the future, TS+TW. TW is a constant. For example, it might be set to 5 seconds. In that example the service would look for potential zone violations for a vehicle from the time of its state report to 5 seconds in the future from that report.

### Vehicle Linear Trajectory

The Zone Alert Service computes linear trajectories. More specifically, we define a vehicle’s linear trajectory as the line (segment) from the vehicle’s reported position to its position at the end of its time window, as if the vehicle were to not accelerate and instead remain on its current heading, climb, and ground speed during the time window.

Naturally, the vehicle may turn, climb, or change speed during this time, but the service does not consider such acceleration, even if it is reported or is directly inferable from the sequence of past vehicle state reports.

### Present Time

A remaining important consideration is time of a alert service, TP or ‘present time’. This is the time when a vehicle receives a state report is received by the alerting service and alerts are computed. TP will be after the time of the state report from the vehicle, TS. On a local or highly reliable system, TP will be very close to TS and will be negligible. However, on a distributed application of a system like OpenUxAS, TS can be far earlier than TP, and so it is important to compute alerts with this real time difference in mind.

### Zone Merging Semantics

Zones received as declarations from other services are defined as 3-dimensional polyhedral. They have a maximum and minimum altitude in the vertical (altitude) plain and a polygon boundary in the horizontal plane.

Ideally, a zone alerting system would alert against these original zones. However, the original zones are not used for two reasons.

- OpenUxAS’s route planning services adjusts the geometry of zones by merging overlapping zones of the same type and by adjusting their geometry with some of (but not all of!) their declared padding. It is useful for the zone alert system to alert for violations of the same geometry as that which is applied in route planning. Therefore, zone alerting must apply merged and modified zone geometry.

- Keep-in zones are applied under the assumption that a vehicle must stay inside any declared keep-in one zone that the vehicle is within at mission start. Furthermore, staying within such a keep-in zone includes staying within the original zone OR any overlapping zone. A solution to this problem is to consider keep-in zones to be implicitly merged by overlap.

The existing OpenUxAS merging semantics performs 2D zone merging. All zone polygon boundaries in the horizontal plane are merged by overlap. Altitude limits of zones (maximum and minimum altitudes forming a 3D polyhedron) are not considered. The resulting merged zones form polygon boundaries in the horizontal plane and disallow and include all altitudes within those boundaries.

Therefore, Zone Alerting applies the above 2D merging semantics and the resulting zones against which it alerts are not the same as the original declared zones.

For this reason,

- The Zone Alert Service re-declares merged zones, presenting the resulting merge 2D geometry to the OpenUxAS service (any listeners)

- When a Zone Alert is declared, it is declared against these zones. The IDs of these ‘merged zones’ are used so that services applying the Zone Alert Service can understand the geometries of zones for which alerts are issued.

### Zone Violation Semantics

### Zone violations have the following semantics against merged zones. (See above for information about merged zones):

- Keep-Out Zone Violation: A vehicle is in violation with a merged keep-out zone if and only if the position of the vehicle is in or on the polygonal boundary of the zone in the horizontal plane.

- Keep-In Zone Violation: A vehicle is in violation with a merged keep-in zone if and only if the vehicle was in or on the polygonal boundary of the zone in its initial reported position to OpenUxAS and its current position is outside of that zone’s polygonal boundary.

In addition, the Zone Alert System makes imminent zone violation alerts. It does so using the reported immediate velocity of a given vehicle state report. When a vehicle issues a state report, it issues the position of the vehicle and the current linear velocity of the vehicle (along with other information (acceleration, etc.) The Zone Alert System ignores acceleration and path interpretation and considers whether a projection of a vehicle from its reported position along vector of its immediate velocity will violate a zone within a given lookahead time.

The lookahead time is set when the service is initiated and is constant.

Imminent violations are defined and reported as follows:

- Imminent Keep-Out Zone Violation: A vehicle is in imminent violation of a merged keep-out zone if the vehicle is not currently in violation with the keep-out zone and one or more positions on the displacement vector from the reported vehicle position along the vehicle’s reported linear velocity vector to the position that would be occupied at lookahead time (from the time reported in the vehicle state report)  is on or within the keep-out zone polygon in the horizontal plane.

- Imminent Keep-In Zone Violation: A vehicle is in imminent violation of a merged keep-in zone if the vehicle was in or on the keep-in zone’s polygon boundary in the horizontal plane in its first published vehicle state report, the vehicle is not currently in violation of the keep-in zone  and one or more positions on the displacement vector from the reported vehicle position along the vehicle’s reported linear velocity vector to the position that would be occupied at lookahead time (from the time reported in the vehicle state report) is outside the keep-in zone polygonal boundary in the horizontal plane.

### What Operating Region Zones to Consider Against What Vehicles

Zones are defined independent of operating regions. An operating region is a collection of zones and prescribed region in which vehicles can operate. Each vehicle is defined as operating within such a region. However, different vehicles can be defined to be operating in different, overlapping regions. Furthermore, zones are defined globally and potentially shared between regions.

However, after discussion with stakeholders, it was decided that alerts will be issued against a vehicle for any zone violation, and users of the service should decide which to consider and which to ignore.

### Reporting Violations for all Received Vehicle State Messages in Order Received

It is useful to report present violations that are occurring at the vehicle state report time TS, even if the state report is received at a much later time. Such alerts can detect and log past zone violations under patchy communication conditions or significant server load. For present simplicity the service shall compute on all received vehicle state messages in the order received, regardless of staleness.

### Semantic Consistency with Other Zone Services

OpenUxAS contains ‘padding’ parameters in zone definitions, utilized to make zones account for a buffer region. Keep in zones are intended to be shrunk by this padding and keep-out zones enlarged by it. The purpose, we infer, is to allow for point-like vehicle representations to account for vehicle size by buffering zone boundaries. But there may be some other use case.

During reverse engineering this code in another project, it was observed that different implemented computations on zones differed in use of this padding information. For example, a padding method might bezel edges of grown keep-out zones. Another might leave flat edges on grows zones. This example of interpretative difference has been observed in the two existing route planning service implementations of zone padding.

Therefore, it is critical that the Zone Alert Service consistently interpret announced zones with padding in the manner of other services. Otherwise, operational inconsistency and advice can occur between services. For example, if a Route Planning Service and a Zone Alert Service pad zones using different edge semantics, Zone Alert Service might alert to violations of keep-out zones on paths planned by the Route Planning Service.

Some services, as mentioned previously, do not apply declared zone buffering. They ignore it for some zone types.

Therefore, it is critical that the Zone Alert Service apply zones in the same manner as applied by other services, so that detecting and alerting are consistent with those services’ use of zone information. As a result, the Zone Alert Service will rely on a Route Planning Service’s definition of zone construction and padding from zones announced to the services.

To achieve this, we will rely on existing polygon zone merge functionality from other services as it exists and is approved. For the moment we may need to copy this functionality so as to not refactor existing services. This is a trade-off and introduces tech debt in duplicate functionality that might later diverge. It is recommended that a zone merging function be common and shared between all services for semantic consistency.

Resulting Solution Behavior Requirements

Top-Level requirements are specified in the requirements spreadsheet in the Specification/Requirements folder.

The following requirements are derived from the mandate requirements and the chosen solution approach. These are described as behavioral use cases in following sections.

- Initialization

  - The service shall require as a configuration parameter a time window, defined in seconds, for imminent violation lookahead.

  - The service shall report and error and terminate if an acceptable time window is not defined in configuration parameters.

- Zone Merging

  - Behaviors

    - The alert service shall merge all zones declared by mission services such that all sets of overlapping zones of the same type (keep-in or keep-out) become a single zone with a new id.

    - The alert service shall declare all resulting merged zones with their new id and polyhedron boundary

    - The merge semantics in the x-y plane shall be an outside edge merge that removes all self-contained holes and includes them in the polygon. See polygon merge function for details.

    - The merge semantics in the z plane shall be to automatically include all altitudes in all zones. Altitude boundaries are discarded.

    - The merge shall announce the new merged zones and their ids when the mission system declares it is done declaring.

  - Usage Note

    - The resulting merge semantics mean that if initially declared keep in zones overlap so as to leave holes (such as forming a ring of keep-region around a central undefined area), the holes will disappear in the merged polygon. A separate keep-out zone should be declared for such intended holes.

    - Any system that wants to understand what zones have been violated, geometrically, should listen for merged zone declarations and record the zone polyhedral and their ids.

    - For the applied merging solution, merging is not associated nor transitive. Order matters in zone merging outcomes.

- Input Data

  - When a vehicle emits a state message, the alert service shall consume it to detect existing and imminent zone alerts

- General Zone Alert

  - Behaviors

    - An emitted zone alert shall contain the vehicle id and the zone Id of the vehicle and merged zone in a detected existing conflict.

    - An emitted zone alert shall contain the cartesian 3D position of the conflict in meter units.

    - An emitted zone alert shall contain the time at which the detected conflict occurs.

    - An emitted zone alert shall contain the time at which the alert was computed by the alert service.

  - Note

    - Time unit is defined by OpenUxAS

- Existing Zone Violation Alerts

  - Behaviors

    - When the zone alert service receives a vehicle state message, it shall emit an existing zone alert for each and every and only merged zones with which the vehicle is in existing violation at its reported position and time.

    - When an existing zone alert is emitted, its reported time of violation shall be the time of state reported in the vehicle state report.

    - When an existing zone alert is emitted, its reported position shall be the position of the vehicle in the vehicle state report.

    - A vehicle position shall be computed as in existing violation with a merged keep-out zone if and only if the reported vehicle position is in or on the polyhedron of the merged keep-out zone.

    - A vehicle position shall be computed as in existing violation with a merged keep-in zone if and only if the vehicle was in or on a merged keep-in zone in its fist received	 position state report and is no longer in or on the same merged keep-in zone.

- Imminent Zone Violation Alerts

  - Behaviors

    - When a zone alert service receives a vehicle state, it shall emit an imminent zone alert for each and every and only merged zones with which the vehicle is in imminent violation over its future projected linear trajectory.

    - The future projected linear trajectory of a vehicle, given its state report, is parametric line segment defined by the state report’s vehicle position and instantaneous linear velocity over the imminent time window.

    - The imminent time window for a vehicle state report is the time interval from but not including the vehicle state report time to and including the vehicle state report time summed with the time window defined in system service configuration.

    - Imminent Zone Violation shall be detected for a vehicle and merged keep-out zone if and only if the vehicle is not in existing keep-out violation with the merged zone and any point on the vehicle’s future projected linear trajectory is in or on the keep-out zone polyhedron.

    - imminent zone violation shall be detected for a vehicle and merged keep-in zone if and only if the vehicle was in a merged keep-in zone at its first state report, the current position of the state report is not in existing zone violation with the merged keep-in zone of the mission and a point on the projected linear trajectory lies outside the merged keep-in zone polyhedron.

    - Imminent Zone violation time and position shall be computed as the time and position of the earliest point on the future projected linear trajectory in violation with the zone.

in the horizontal plane.

Service Parameters

The service is parameterized when the service is first instantiated with the following parameters:

- Lookahead time window subscriber TW.